Record-DUT vi

DUT Mic Recording via HTTP handler

http://(DUT IP):1200/operation/streamaudio?source=mic&mic=(mic qty)&applycoeffs=no&duration=(recording duration)&header=no

DUT IP: 169.254.1.1

mic qty : Binary to Decimal (8 mics = 11111111 = 127, 4 mics = 00001111 = 15)

recording duration: total record time

Input:

record config (cluster): DUT recording configuration

sweep duration (double): stimulus sweep duration

Mics Mapping Index: Mic sequence index to physical label (0 - 7)

Output:

waveform out (array): recorded waveform out

waveform Y (array d): waveform Y data