curvenaming vi

- Automatically name input data for the Mic test or Speaker test.

- Mic test naming will start from Mic[1…n]

- Speaker test will naming with Left[1…n] & Right[1…n]

- If Ref mic enabled, the curve name with Ref Mic will be added into the output

- GS/DUT will add the GS/DUT into the curve name

Naming format:-

FR - Fundamental GS Mic1

FR - Fundamental GS Mic2

FR - Fundamental GS Mic3

FR - Fundamental GS Mic4

FR - Smoothed GS Mic1

FR - Smoothed GS Mic2

FR - Smoothed GS Mic3

FR - Smoothed GS Mic4

FR - Fundamental DUT Mic1

FR - Fundamental DUT Mic2

FR - Fundamental DUT Mic3

FR - Fundamental DUT Mic4

FR - Smoothed DUT Mic1

FR - Smoothed DUT Mic2

FR - Smoothed DUT Mic3

FR - Smoothed DUT Mic4THD - DUT Mic1

THD - DUT Mic2

THD - DUT Mic3

THD - DUT Mic4

Input:

Mic? (Boolean): define if this is mic test or speaker test

Ref Mic (Boolean): enable if the ref mic data was recorded

dBSPL? (Boolean): Value convert to dB SPL (re to 20u) instead of dB V (re to 1)

GS? (Boolean): define if this is GS or DUT data

DUT Curve in (Cluster):

Ref Mic Curve Data (Cluster):

Output:

Curve out (Cluster):

Front Panel: