Read SoundCard Calibration vi

- Import Calibration data from the Calibration process (Calibrations.ini)

- Calibration data included: Input & output peak value, Amp Gain, and Microphone Sensitivity

Calibration.exe: Application to get and store calibration values into Calibrations.ini

Output from Calibration exe: Calibrations.ini

[Input]

Microphone (USB Advanced Audio Device) L = 1

Microphone (USB Advanced Audio Device) R = 1

[Output]

Speakers (USB Advanced Audio Device) L = 1.092

Speakers (USB Advanced Audio Device) R = 1.092

[Amp]

Speakers (USB Advanced Audio Device) - Amp L = 6.32

Speakers (USB Advanced Audio Device) - Amp R = 6.32

[Mic]

Microphone (USB Advanced Audio Device) - Mic L = 1

Microphone (USB Advanced Audio Device) - Mic R = 1

Input:

In/Out (string): Select input or output

Soundcard Type (string): select the type of soundcard driver (WDM/MME, ASIO)

Soundcard Name (string): soundcard name

Selected Channel (int): channel number

Channel Name (string Array): for WDM/MME soundcard, the name will be L or R, for ASIO soundcard, the channel name can be retrieved from the soundcard info (input name or output name)

Output:

Output Calibration (Cluster):

Input Calibration (Cluster):

Front Panel: